Assignment 8 Solutions

1.

Ld/St

| Opcode | Rs | Rt | Address Offset |
| --- | --- | --- | --- |

Opcode is set to the opcode for either load or store.

Rt specifies the register which with get the value from memory for load instructions, or will provide the value to memory for stores.

Rs specifies a register whose value will be added to the offset value to provide the address to memory.

Arithmetic/Logic (register only operands)

| Opcode | Rs | Rt | Rd | shamt | Funct |
| --- | --- | --- | --- | --- | --- |

Opcode is set to the opcode for an arithmetic operation.

Rd specifies the register which with get the value from the alu operation

Rs and Rt specify registers whose values will be used in the operation.

Shamt is used to specify the amount to shift in a shift instruction.

Funct is used to determine which arithmetic or logical operation to use.

Arithmetic/Logic (immediate value)

| Opcode | Rs | Rt | Immediate Value |
| --- | --- | --- | --- |

Opcode is set to the opcode for the specific arithmetic with immediate instruction

Rt specifies the register which with get the value from the alu operation.

Rs specifies a register whose value will be used as an operand in the alu operation.

Immediate Value provides the other operand to the alu.

Branch

| Opcode | Rs | Rt | Address |
| --- | --- | --- | --- |

Opcode is set to the opcode for the branch instruction.

Rs and Rt specify the registers whose values will be compared in the branch instruction.

Address specifies the address (relative to the PC) that the program will jump to if the branch is successful.

Jump

| Opcode | Target Address |
| --- | --- |

Opcode is set to the opcode for jump.

Target Address specifies the address of the instruction the program will jump to.

2. a)

start:

lw $t0, mask		//$t0 = 0xfffff83f, 1….100000111111

lw $s0, shifter		//$s0 = sll instruction

and $s0,$s0,$t0	//clears the shamt field in the sll instruction stored in $s0

andi $s2,$s2,0x1f	//clear all but the last 5 bits in $s2

sll $s2,$s2,6		//move those bits to the position that corresponds to shamt

or $s0,$s0,$s2		//set the shamt field in $s0 as the value from $s2

sw $s0, shifter		//store the sll instruction back into memory

Self-modifying code such as this is a bad idea because you are changing code generated by the compiler and will make your program machine dependent.

| Instr | Format | Op | Rs | Rt | Immed or Rd/Shamt/Func |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lw $t0, mask | I | 35 | 0 | $t0 | mask |  |  |  |  |
| lw $s0, shifter | I | 35 | 0 | $s0 | shifter |  |  |  |  |
| and $s0,$s0,$t0 | R | 0 | $s0 | $t0 | $s0 |  | 0 |  | 32 |
| andi $s2,$s2,0x1f | I | 12 | $s2 | $s2 | 31 |  |  |  |  |
| sll $s2,$s2,6 | R | 0 | 0 | $s2 | $s2 | 6 |  | 0 |  |
| or $s0,$s0,$s2 | R | 0 | $s0 | $s2 | $s0 | 0 |  | 37 |  |
| sw $s0, shifter | I | 43 | 0 | $s0 | shifter |  |  |  |  |

3.  A possible name could be srreg $s0, $s1, for “shift right registers”.  The instruction shifts both $s0 and $s1 right one bit, shifting the least significant bit of $s0 into the most significant bit of $s1.  A diagram for the action is shown below.

$s0

$s1

LSB$s1

LSB$s0

0

4.

li $r4, 0 	# sum = 0

li $r5, 0 	# i = 0

li $r6, 10 	# n = 10

li $r7, 0x04000000 # address of array

loop:

beq $r5, $r6, end	# end if i = 10

add $t0, $r5, $r7	# calculate address of array[i]

lw $t0, 0($t0)		# temp = array[i]

add $r4, $r4, $t0	# sum = sum + temp

addi $r5, $r5, 1	# increment i

j loop			# jump to loop

end:

5.

Only F is true.