Assignment 10 Solutions

- a)

lis r2, 0x0000		;set upper 2 bytes of address

ori r2, r2, 0x66b0	;set lower two bytes of address

li r3, 25		;r3 = 25

stw r3, 0(r2)		; mem[r2] = 25

b)

i. r20 = 0x00000083

ii. r21 = 0x0000AC57

iii. r22 = 0x30002012

iv. Address		Memory Contents

30002000 	00 FE 25 24 80 02 EE 05

c)

| Program Fragment |  | After the instruction executes: |  |
| --- | --- | --- | --- |
|  | addi r3,r3,4 |  |  |
|  | bl func1 | LR  0x00005000 (return address) |  |
|  | b end |  |  |
| func1: | mflr r8 |  |  |
|  | addi SP, SP, -4 | SP = 0x00007FFC_____ |  |
|  | stw r8, 4(SP) | SP = 0x00007FFC_____ | M[SP+4] = _0x00005000____ |
|  | stw r3, 0(r5) | r5 = _0x00003000____ | M[r5+0] = _0x00000004____ |
|  | bl func2 |  |  |
|  | lwz r8, 4(SP) | r8 = __0x00005000__ |  |
|  | addi SP, SP, 4 |  |  |
|  | mtlr r8 |  |  |
|  | blr |  |  |
| func2: | addi r3,r3,1 |  |  |
|  | stw r3,0(r10) |  |  |
|  | blr |  |  |
| end: |  |  |  |

d) addi rD, rD, -1

- a)

  - lbz r30, 10(r20)

  - lhz r30, 10(r20)

| Array | Element Size (bytes) | Total Size (bytes) | Start Address | Element i Address |
| --- | --- | --- | --- | --- |
| S | 2 | 14 | xS | xS + 2i |
| T | 4 | 12 | xT | xT + 4i |
| V | 4 | 32 | xV | xV + 4i |
| W | 4 | 16 | xW | xW + 4i |

  - lwz r30, 4(r20)

  - lwz r30, 12(r20)

| Offset | 0 | 4 | 12 | Total Size |
| --- | --- | --- | --- | --- |
| Field | c | i | v | 16 bytes |

| Offset | 0 | 1 | 4 | 8 | 12 |
| --- | --- | --- | --- | --- | --- |
| Contents | c | XXX | i[0] | i[1] | v |

where XXX denotes a 3-byte gap

iv.

struct P2 { int i; char c; char d; int j; };

| Field: i, c, d, j | Total Size (bytes) | Structure Alignment |
| --- | --- | --- |
| Offset: 0, 4, 5, 8 | 12 | 4 bytes |

struct P3 { short w[3]; char c[3]; };

| Field: w, c | Total Size (bytes) | Structure Alignment |
| --- | --- | --- |
| Offset: 0, 6 | 10 | 2 bytes |

struct P4 { short w[3]; char * c[3]; };

| Field: w, c | Total Size (bytes) | Structure Alignment |
| --- | --- | --- |
| Offset: 0, 8 | 20 | 4 bytes |