# Assignment 12

- Problem 8.2 in Brown and Vranesic (20 pts)

Derive a circuit that realizes the FSM defined by the state-assigned table in Figure p8.1 using JK flip-flops.

| Present State y2y1 | Next State w=0 w=1 Y2Y1 Y2Y1 | Output z |
| --- | --- | --- |
| 00 01 10 11 | - 11 - 00 - 00 10 01 | 0 0 0 1 |

Figure p8.1. State-assigned table for problem 8.2

- Problem 8.5 and 8.6 in Brown and Vranesic

  - Derive a minimal state table for a single-input and single-output Moore-type FSM that produces an output of 1 if in the input sequence it detects either 110 or 101 patterns. Overlapping sequences should be detected.

  - Repeat the problem for a Mealy-type FSM.

  - Design and circuits using D flip flops in each case.

- Problem 8.9 in Brown and Vranesic

A sequential circuit has two inputs w1 and w2, and an output, z. Its function is to compare the input sequences on the two inputs. If w1= w2 during any four consecutive clock cycles, the circuit produces z=1; otherwise, z=0. For example

w1: 0110111000110

w2: 1110101000111

z:  0000100001110

Derive a suitable circuit.

- Write Verilog code for the FSM described in problem 3 and use waveform to verify your code’s correctness.

- Design a three-bit counter like circuit controlled by the input w. If w=1, then the counter adds 2 to its contents, wrapping around if the count reaches 6 or 7. Thus if the present state is 6 or 7, then the next state becomes 0 or 1, respectively. If w=0, then the counter subtracts 1 from its contents, acting as a normal down-counter. Use JK flip-flops in your circuit.