Assignment 1 Solutions

- One possible solution is:

a) Numbers 0-9	b)  Alphabets A-F	c)  Integers 21-36

0 = 0000		A = 001		21 = 0001	29 = 1001

1 = 0001		B = 010		22 = 0010	30 = 1010

2 = 0010		C = 011		23 = 0011	31 = 1011

3 = 0011		D = 100		24 = 0100	32 = 1100

4 = 0100		E = 101		25 = 0101	33 = 1101

5 = 0101		F = 110		26 = 0110	34 = 1110

6 = 0110					27 = 0111	35 = 1111

7 = 0111					28 = 1000	36 = 10000

8 = 1000

9 = 1001

- a) 7 bits

b) 12 bits

c) 15 bits

d) 21 bits

- a) 24-1 or 15

b) 212-1 or 4095

c) 224-1 or 16,777,215

- a) 10

b) 18

c) 62

d) 128

e) 63

- a) 110

b) 1101

c) 1101111

d) 10010011

e) 111111111

- If you use each finger to represent a binary digit, at total of 210-1, or 1023 laps can be counted.  However, it may be argued that keeping track of binary numbers whilst jogging is not convenient, and thus only ten laps may be easily counted.

- A slide rule is a tool for performing multiplication and division based on addition.  It makes use of a property of logarithms, specifically, the fact that the log of the product of two numbers is equal to the sum of their logarithms (log (x*y) = log x + log y).  The numbers on a slide rule are arranged along a line such that the distance between a number and the origin is directly proportional to its log.  So, to multiply two numbers on a slide rule, align the origin of one bar with the first operand on the second.  The location of  the second operand corresponds to the sum of the logs, and will be at the position of the product on the first bar.

- Structure 1: tm*(n+1) + ta; structure 2: n*(tm+ta)