Assignment 4 Solutions

1.  module prob1 (x1, x2, x3, f);

input x1, x2, x3;

output f;

not(nx1, x1);

not(nx2, x2);

not(nx3, x3);

and(w1, x1, x2, x3);

and(w2, x1, nx2, nx3);

and(w3, nx1, x2, nx3);

and(w4, nx1, nx2, x3);

or(f, w1, w2, w3, w4);

endmodule

2.  Simplified circuit (using K-map or Boolean algebra): f = x1’.x3+x2.x3’+x1.x2’

module prob2 (x1, x2, x3, f);

input x1, x2, x3;

output f;

not(nx1, x1);

not(nx2, x2);

not(nx3, x3);

and(w1, nx1, x3);

and(w2, x2, nx3);

and(w3, x1, nx2);

or(f, w1, w2, w3);

endmodule

3.  module fulladd (a, b, cin, sum, cout);

input a, b, cin;

output sum, cout;

assign sum =  a ^ b ^ cin;

assign cout = (a & b) | (a & cin) | (b & cin);

endmodule

OR

module halfadd (a, b, sum, cout);		module fulladd(a, b, cin, sum, cout);

input a, b;					input a, b, cin;

output sum, cout;				output sum, cout;

wire w, c1, c2;

xor(sum, a, b);					halfadd ha1 (a, b, w, c1);

and (cout, a, b);				halfadd ha2 (w1, cin, sum, c2);

or(cout, c1, c2);

endmodule					endmodule

- A) a)

| x1 | x2 | x3 | f |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

b) We need 3 NOT gates, 4 3-input AND gates, and one 4 input OR gate.  It takes 2	transistors per NOT gate, 8 per AND gate, and 10 for the OR gate.  Thus, total is

3*2 + 4*8 + 10 = 48 transistors.

B) a) Analyzing the multiplexer circuit, g = x1.(x2.x3 + x2’.x3’) + x1’.(x2.x3’ + x2’.x3),

Simplifying gives g = x1.x2.x3 + x1.x2’.x3’ + x1’.x2.x3’ + x1.x2’.x3 which is   equivalent to the sum of products obtained from P3.1.

b) We need 1 NOT gate and 3 MUXes, at 2 transistors for a NOT gate and 6 per MUX, this gives at total of 2 + 3*6 = 20 transistors  (using transmission gates for multiplexer,  alternately, using 1 NOT, 2 AND, and 1 OR for MUX => 20/MUX, so 62 total).

- a)

| x1 | x2 | x3 | x4 | f |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 |

b) f = x1’x3’x4’ + x2’x3’x4’

We would need 4 NOT gates, 2 3-input ANDs, and 1 2-input OR, so

4*2 + 2*8 + 6 = 30 transistors.

- a)

| a | b | c | d | f |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

b + c)

| ab |  |  |  |  |
| --- | --- | --- | --- | --- |
| cd | 00 | 01 | 11 | 10 |
| 00 | 1 | 0 | 1 | 0 |
| 01 | 0 | 1 | 0 | 1 |
| 11 | 1 | 0 | 1 | 0 |
| 10 | 0 | 1 | 0 | 1 |

Sum of products

f = a’b’c’d’ + a’b’cd + a’bc’d + a’bcd’ + ab’c’d + ab’cd’ + abc’d’ + abcd.

Product of sums

f = (a+b+c+d’).(a+b+c’+d).(a+b’+c+d).(a+b’+c’+d’).(a’+b+c+d).(a’+b+c’+d’).(a’+b’+c+d’).	(a’+b’+c’+d)

d)		circuit for (b)

circuit for (c)

e)		circuit for (b)

circuit for (c)

f)

g)