CprE 381 Homework #7 Solution

- In general, the answer is no.  With very large memories, the cost of address decoding becomes significant.  Also, using a large amount of SRAM will greatly increase energy usage and require more complicated cooling.  However, either answer, if argued properly, is acceptable.

- A program that accesses a large amount of memory (e.g. a large array) with a large stride would have little temporal and spatial locality.  For example:

int array[10000][100000];

int i, j, stride = 1000;

for (i in 0 to 10000)

for( j in 0 to 10000, j = j+stride)

sum = sum + array[i][j];

- A program that repeatedly branches between a few locations, spaced far apart in memory.  For example:

while(1)

{

function1Call(params);

function2Call(params);

function3Call(params);

}

- All accesses miss except for the second reference to 11.  Final contents are:

| 0 | 48 |
| --- | --- |
| 1 |  |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 21 |
| 6 | 6 |
| 7 |  |
| 8 |  |
| 9 |  |
| 10 |  |
| 11 | 11 |
| 12 |  |
| 13 | 13 |
| 14 |  |
| 15 |  |

- Row major should be faster, since memory is laid out in rows first.  This should show a large effect of memory layout.

- Attach a ‘1’ to the tag coming from the request, and compare it with the stored tag concatenated with the valid bit.  Then only the equality comparison is needed.

- Access time for a) is 16*10+16*1 = 176 cycles.  Access time for b) is 4*10+4*1 = 44 cycles.  Assuming each bank is 4 words wide, access time for c) is 1*10+16*1 = 26 cycles.

- AMAT = 1*2ns + .05*20*2ns = 4ns.

- Since we can’t shorten to fractions of cycles, cache access will have to take 2 cycles now, so AMAT = 2*2ns + .03*20*2ns = 5.2ns, which is not a good trade off.

AMAT for the different organizations is:

| Block size (words) | Associativity | Instruction miss rate | Data miss rate |
| --- | --- | --- | --- |
| 1 | 1 | 2.11 | 1.71 |
| 4 | 1 | 1.82 | 1.772 |
| 4 | 2 | 1.956 | 1.924 |
| 4 | 4 | 1.956 | 1.924 |

The best organization for I cache is a direct mapped cache with 4 word blocks.  The best organization for D cache is a direct mapped cache with 1 word blocks.

- Again, all accesses miss except for the second reference to 11.  Final contents are:

| 0 | 48 | 64 |
| --- | --- | --- |
| 1 |  |  |
| 2 | 2 |  |
| 3 | 11 | 27 |
| 4 | 4 |  |
| 5 | 21 | 13 |
| 6 | 22 | 6 |
| 7 |  |  |

- Hits are highlighted: 2, 3, 11, 16, 21, 13, 64, 48, 19, 11, 3, 22, 4, 27, 6, 11

Final contents are:

| 0 | 24 | 25 | 26 | 27 |
| --- | --- | --- | --- | --- |
| 1 | 8 | 9 | 10 | 11 |
| 2 | 20 | 21 | 22 | 23 |
| 3 | 4 | 5 | 6 | 7 |

- The total bits for index are log2 [S/(B*A)]

The total bits for the cache are the total of the following:

Bits for data = S*8

Bits for tag = (k - index_bits – b)*S/B

Bits for valid = 1*S/B

- Cache 1 spends .04*7*I + .06*.5*7*I = .49*I

Cache 2 spends .02*10*I + .04*.5*10*I = 0.4*I

Cache 3 spends .02*10*I + .03*.5*10*I = 0.35*I

Therefore processor 1 spends the most cycles on cache misses.

- With a direct mapped cache and stride 256, the loop will reference addresses 0 and 256, both of which are mapped to location 0 in the cache, so the cache will miss every time, leading to a miss rate is 100%.

With a direct mapped cache and stride 255, the loop will reference addresses 0, 255, and 510, which are mapped to locations 0, 6, and 7.  This will lead to only the 3 initial misses.  Since 30,000 references are made, the miss rate is 0.01%.

With a 2-way set associative and stride = 256, the data for 0 and 256 can now be placed in the cache, so only 2 misses of the 20,000 references occur.  Thus miss rate is 0.01%.

Lastly, with 2-way set associative and stride = 256, the addresses 0, 255, and 510 will be mapped to 0, 3 and 2.  So, like before, only 3 misses occur in 30,000.  Miss rate is 0.01%.