CprE 381 Homework #4

Reading Assignment: Chapters 4 and 5

P1. (50 points) Draw a complete multi-cycle data path for the machine you are designing in lab using the following details. Design a control machine to generate all control signals.

Instruction Format

| J Format | OPCODE | ADDRESS |
| --- | --- | --- |
|  | 31:26 | 25:0 |

| I Format | OPCODE | RS | RT | IMM/OFFSET |
| --- | --- | --- | --- | --- |
|  | 31:26 | 25:21 | 20:16 | 15:0 |

| R Format | OPCODE | RS | RT | RD | Unused | Unused |
| --- | --- | --- | --- | --- | --- | --- |
|  | 31:26 | 25:21 | 20:16 | 15:11 | 10:6 | 5:0 |

OPCODES

| Opcode | Instruction | Format |
| --- | --- | --- |
| 000000 | No Op | - |
| 000001 | add | R |
| 000010 | sub | R |
| 000011 | and | R |
| 000100 | or | R |
| 000101 | slt | R |
| 000110 | lw | I |
| 000111 | sw | I |
| 001000 | beq | I |
| 001001 | bne | I |
| 001010 | addi | I |
| 001011 | andi | I |
| 001100 | ori | I |
| 001101 | slti | I |
| 001110 | j | J |

P2. (10 points) Suppose our register file in multi-cycle implementation has only one read port. Redraw the data path in Problem 1 and write register transfer level descriptions of our instructions in Problem 1 again.

P3. (40 points) Suppose we would like to add all shift class instruction based on our shifter design. The format will be similar to I format with bit 15-12 specifying the shift type. Bits 11-0 are unused. Suggest the changes in data path and control.