CprE 381 Homework #5

Reading Assignment: Chapters 6

P1. (50 points) Draw a complete pipeline data path for the machine whose instruction set is shown below with 16 registers. Design a control machine to generate all control signals.

Instruction Format

| J Format | OPCODE | ADDRESS |  |  |
| --- | --- | --- | --- | --- |
|  | 15:12 | 11:0 |  |  |
| I/LW/SW/BEQ/BNE Format | OPCODE | IMM/OFFSET | RT | RS |
|  | 15:12 | 11:8 | 7:4 | 3:0 |
| R Format | OPCODE | RD or SHIFT | RT | RS |
|  | 15:12 | 11:8 | 7:4 | 3:0 |

OPCODES

| Opcode | Instruction Operation |
| --- | --- |
| 0000 | Add |
| 0001 | And |
| 0010 | Or |
| 0011 | Slt |
| 0100 | Lw |
| 0101 | Sw |
| 0110 | Beq |
| 0111 | Bne |
| 1000 | Addi |
| 1001 | Andi |
| 1010 | Ori |
| 1011 | Slti |
| 1100 | Sub |
| 1101 | Shift |
| 1110 | Jr |
| 1111 | Jal |

P2. (30 points) Download the animator provided to you from the course web page, unzip it and play with the animator interface provide to you and answer the following question. For each of the traces provided, how many clock cycles it takes to finish the first 10 instructions (that is when the first non-noop instruction is fetched to the 10th instruction is completed). In each case, how many useful (non-noop or bubbles) instruction are executed. In each case, give a brief description of what mechanism is employed for correct execution and what saving is achieved in terms of clock cycles.

P3. (20 points) PROJECT INSTRUCTION SET: As part of this course we will be implementing a pipelined processor. You and your partner should choose a small set of instructions for a microprocessor. Your project should be designed along the labs you did in this class or modification of them. The instruction set should be upgraded from what was done in the labs. Submit the instruction set along with the format of each instruction you plan to implement. Give a brief justification for why you chose this instruction set. Both you and your partner should sign the submitted work. This is an agreement between two of you and me that you will be implementing this instruction set. You will be graded on the quality of presentation in terms of completeness of the description.