Cpre 381 - Homework 2 Solution

module converter(datain, dataout, clk, done, rst);

input [31:0] datain;

input clk, rst;

output [31:0] dataout;

output done;

reg [31:0] value, dataout;

reg [7:0] count;

reg done;

wire [31:0] shiftOut;

wire shiftOutBit;

//state variables

reg [1:0] current, new;

//define allowable states

parameter [1:0] RESET = 2'b00, INIT = 2'b01, SHIFT = 2'b10, DONE = 2'b11;

//shift32bit (In, ShiftIn, DV, MS, sel, Out, shift_out);

shift32bit s(value, , , , 4'b0001, shiftOut, shiftOutBit);

always @(current or shiftOutBit)

begin

case (current)

RESET:

begin

new = INIT;

done = 1'b0;

dataout = 0;

end

INIT:

begin

new = SHIFT;

done = 1'b0;

dataout = 0;

end

SHIFT:

begin

if (shiftOutBit == 0)

new = SHIFT;

else

new = DONE;

done = 1'b0;

dataout = 0;

end

DONE:

begin

new = DONE;

done = 1'b1;

dataout = {1'b0,count, value[31:9]};

end

endcase

end

always @(posedge rst or posedge clk)

begin

if (rst == 1)

current = RESET;

else

begin

if (current == INIT)

begin

value = datain;

count = 8'b10001111;

end

else if (current == SHIFT)

begin

value = shiftOut;

count = count - 1;

end

current = new;

end

end

endmodule

This module converts numbers in fixed point format to IEEE floating point format.  The most significant 16 bits of the input give the integer portion, and the least significant 16 bits give the fractional portion.  The output will be the number converted to IEEE 754 floating point format.

Assuming all instructions of class A, P1 will have a peak rate of 4 billion instructions per second, since it runs 4 billion cycles per second and can complete 1 instruction per cycle.  P2 will have a peak rate of 3 billion instructions per second, since it runs 6 billion cycles per second but can complete only 1 instruction per every 2 cycles.

500,000,000 multiplies were replaced.  Total cycles is given by:

Cyclestotal=Cyclesmult+Cyclesother=Imult∗CPImult+Cyclesother

For the first case, this gives:

10sec*1GHz=Imult∗4+Cyclesother

For the second case, this gives:

9sec*1GHz=2∗Imult∗1+Cyclesother

Solving for Imult gives Imult = 500,000,000.

We can assume 100 original instructions to make calculation easier.

First find execution time on both MFP and MNFP:

ExecutionTime=CyclesClockRate=CPIi×CiClockRate

ExecutionTimeMFP=10∗6+15∗4+5∗20+70∗21000MHz=3.6×10−7

ExecutionTimeMNFP=(10∗30+15∗20+5∗50+70)∗21000MHz=1.84×10−6

Now find MIPS:

MIPSMFP=InstructionCountExecutionTime×106=1003.6×10−7∗106=277.78

MIPSMNFP=InstructionCountExecutionTime×106=9201.84×10−6∗106=500

Again, we’ll assume 100 instructions.

CPI can be calculated as follows:

CPI=TotalCyclesInstructions=CPIi×CiInstructions

CPIMbase=40∗2+25∗3+25∗3+10∗5100=2.8

CPIMopt=40∗2+25∗2+25∗3+10∗4100=2.45